| Feature | MPU-6050 | MPU-9250 | MPU-9255 |
| --- | --- | --- | --- |
| DOF / Axes | 6-axis (gyro + accel) | 9-axis (gyro + accel + mag) | 9-axis (gyro + accel + mag) |
| Gyroscope | 3-axis | 3-axis | 3-axis |
| Gyro full-scale | ±250/500/1000/2000 °/s | ±250/500/1000/2000 °/s | ±250/500/1000/2000 °/s |
| Accelerometer | 3-axis | 3-axis | 3-axis |
| Accel full-scale | ±2/4/8/16 g | ±2/4/8/16 g | ±2/4/8/16 g |
| Magnetometer | ❌ None | ✅ AK8963, ±4800 µT, 14/16-bit | ✅ AK8963, ±4800 µT, 14/16-bit |
| ADC resolution | 16-bit | 16-bit | 16-bit |
| Interface | I²C only | I²C + SPI | I²C + SPI |
| I²C speed | up to 400 kHz | up to 400 kHz | up to 400 kHz |
| SPI speed | ❌ | up to 1 MHz (20 MHz for sensor reads) | up to 1 MHz (20 MHz for sensor reads) |
| DMP (Digital Motion Processor) | ✅ | ✅ | ✅ |
| FIFO | 1024 bytes | 512 bytes | 512 bytes |
| VDD | 2.375–3.46 V | 2.4–3.6 V | 2.4–3.6 V |
| VDDIO / VLOGIC | 1.71 V–VDD | 1.71 V–VDD | 1.71 V–VDD |
| Operating current | ~3.9 mA (gyro+accel) | ~3.5 mA (9-axis) | ~3.5 mA (9-axis) |
| Gyro noise | 0.005 °/s/√Hz | 0.01 °/s/√Hz | 0.01 °/s/√Hz |
| Accel noise | ~400 µg/√Hz | ~300 µg/√Hz | ~300 µg/√Hz |
| Temp sensor | ✅ | ✅ | ✅ |
| WHO_AM_I value | 0x68 | 0x71 | 0x73 |
| Internal silicon | MPU-6000 family | MPU-6500 + AK8963 | MPU-6555 + AK8963 |
| Package | 4×4×0.9 mm QFN (24-pin) | 3×3×1 mm QFN (24-pin) | 3×3×1 mm QFN (24-pin) |
| Era | ~2012 | ~2014 | ~2014 |

BNO055 9-DOF vs BNO086 9-DOF:

======================================================================

Here's a side-by-side comparison of these two sensor-fusion IMUs. Note they come from different lineages — the BNO055 is pure Bosch, while the BNO08x family (including BNO086) is Bosch MEMS hardware running CEVA/Hillcrest's SH-2 fusion firmware.

| Feature | BNO055 | BNO086 |
| --- | --- | --- |
| Vendor / fusion | Bosch — BSX3.0 FusionLib | Bosch MEMS + CEVA/Hillcrest SH-2 MotionEngine |
| DOF | 9-axis (accel + gyro + mag) | 9-axis (accel + gyro + mag) |
| On-board MCU | ARM Cortex-M0+ | ARM Cortex-M0+ (runs SH-2) |
| Fusion done on-chip? | ✅ Yes | ✅ Yes |
| Quaternion output | ✅ | ✅ |
| Accel full-scale | ±2/4/8/16 g | ±2/4/8/16 g |
| Gyro full-scale | ±125–2000 °/s | ±125–2000 °/s |
| Mag | integrated (Bosch) | integrated (Bosch) |
| Interfaces | I²C, UART, HID-over-I²C | I²C, SPI, UART-SHTP, UART-RVC |
| Fusion output rate | ~100 Hz | up to ~400 Hz (raw); fusion ~100–400 Hz |
| Heading accuracy (dynamic) | ~2.5° typ | ~1–2° typ, generally better |
| AR/VR stabilized rotation vector | ❌ | ✅ |
| Gaming rotation vector | ❌ (no mag-free mode as such) | ✅ |
| Geomagnetic rotation vector | ✅ (the default absolute mode) | ✅ |
| Activity classifier / step counter | step counter only | ✅ classifier, step, significant-motion |
| Tap / shake / pickup detectors | limited | ✅ tap, shake, stability, pickup |
| Calibration | manual-ish, persistent offsets, fiddly | dynamic, mostly automatic |
| VDD | 2.4–3.6 V | 1.65–3.6 V |
| Package | 28-pin LGA, 3.8×5.2×1.13 mm | 28-pin LGA, 3.8×5.2×1.0 mm |
| Comms protocol | register-based (simple) | SHTP packet protocol (more complex) |
| Era / status | ~2015, mature, widely cloned | ~2020, current production |

BNO086 9-DOF vs MPU-9255:

======================================================================
yourself." That distinction matters more than any single row in the table.

| Feature | BNO086 | MPU-9255 |
| --- | --- | --- |
| Class | System-in-package (smart IMU) | Raw 9-axis IMU |
| Vendor / fusion | Bosch MEMS + CEVA/Hillcrest SH-2 | InvenSense/TDK (MPU-6555 + AK8963) |
| Fusion done on-chip? | ✅ Yes — quaternion straight out | ❌ No — you fuse on the host |
| On-board MCU | ARM Cortex-M0+ (runs SH-2) | None (DMP only — limited) |
| DOF | 9-axis | 9-axis |
| Quaternion output | ✅ Native | ❌ (compute yourself) |
| Accel full-scale | ±2/4/8/16 g | ±2/4/8/16 g |
| Gyro full-scale | ±125–2000 °/s | ±250/500/1000/2000 °/s |
| Magnetometer | integrated (Bosch) | AK8963, ±4800 µT |
| Interfaces | I²C, SPI, UART-SHTP, UART-RVC | I²C + SPI |
| SPI speed | up to ~3 MHz | 1 MHz (20 MHz sensor reads) |
| Output / fusion rate | up to ~400 Hz | gyro up to 8 kHz raw; fusion = whatever your host does |
| Heading accuracy (dynamic) | ~1–2° typ (fused) | depends entirely on your fusion code |
| AR/VR & gaming rotation vector | ✅ | ❌ |
| Activity / gesture / tap detect | ✅ | ❌ (basic motion-wake only) |
| Calibration | dynamic, mostly automatic | fully manual (you write it) |
| Comms protocol | SHTP packets (or simple UART-RVC) | plain register reads |
| WHO_AM_I | n/a (product ID via SHTP) | 0x73 |
| VDD | 1.65–3.6 V | 2.4–3.6 V |
| Package | 28-pin LGA, 3.8×5.2×1.0 mm | 3×3×1 mm QFN (24-pin) |
| Era / status | ~2020, current production | ~2014, EOL / not-for-new-design |
| Relative cost | higher | low |

The core trade-off: